Multi-scale Greenhouse Gas Monitoring and Modeling: From Fossil Energy Emissions to Climate Feedbacks

Manvendra K. Dubey

Los Alamos National Laboratory, Los Alamos NM 87545

dubey@lanl.gov

In 2011, LANL deployed a solar tracking high-resolution Fourier Transform Spectrometer (FTS, Bruker 125HR) in Four Corners, NM a semi arid region with two coal-fired power plants, significant fossil energy exploration and a mid-size city.  High-resolution solar spectra in the near infrared and visible are collected every 2 minutes and analyzed using stringent requirements of the Total Column Carbon Observing Network (TCCON) to retrieve columnar green house gases (CO2, CH4, N2O) and pollutants (CO). In situ measurements of CO2, CH4 and CO are made using a Picarro Cavity ringdown spectrometer and NO2, SO2, O3 and particulates are measured using EPA protocols. We also have a meteorological station, a ceilometer to measure boundary layer heights and a Cimel to measure aerosol optical depths. Our system was calibrated using in situ HIPPO vertical profile trace gas measurements taken on 7 June 2011.  We report the power-plant signals and their diurnal cycles and how they depend on the local meteorology. Typically, our FTS observes 2 to 8 ppm increases in CO2 and our in situ Picarro measures increases of 10 to 50 ppm when a power plume is blowing towards our site. In situ CH4 reveal large nocturnal increases of 4-5 ppm (>300%) that together the 0.05-0.09 ppm (3-5%) FTS column increase in the morning suggests a large dispersed source. We hypothesize that this could be seepage of fugitive emissions from extensive gas and coal mining activities in the region. In contrast to CH4 in situ CO2 increases at night are small since the power plant stacks are higher than boundary layer.  We have also developed a plume to regional chemistry-carbon model for Four Corners.  Multi-scale foreword modeling simulations are compared with our column and in situ observations for specific days. We also examine satellite data (CO2 and CH4 from GOSAT and NO2 from OMI) for signals over Four Corners and assess the value added for longer-term trend analysis. We will discuss the deployment of our FTS to Manaus, Brazil in 2014 to monitor the Amazonian carbon-water cycle, and partnerships to expand monitoring in the Arctic for the early detection of carbon release from the warming tundra.