Development of a Supercontinuum Photoacoustic Aerosol Absorption and Albedo Spectrometer for the Characterization of Aerosol Optics

H. Moosmüller1, C. Mazzoleni2, I. Arnold1,3, R. Chakrabarty1, and W. Arnott3

1Division of Atmospheric Sciences, Desert Research Institute, Reno, Nevada, USA

2Department of Physics, Michigan Technological University, Houghton, MI, USA

3Department of Physics, University of Nevada Reno, Reno, Nevada, USA

Email: Hans.Moosmuller@dri.edu

The ongoing development of a novel Photoacoustic Aerosol Light Absorption and Albedo Spectrometer  (PALAAS) for real time, in situ, first principle measurement of aerosol light absorption, scattering, and single scattering albedo spectra is discussed. This instrument utilizes a novel broadband supercontinuum laser covering the solar spectral range from 400 to 2000 nm for simultaneous measurement of aerosol light absorption and scattering coefficients and aerosol single scattering albedo in 32 customizable spectral bands, thereby increasing the number of bands by about one order of magnitude over state-of-the-art three-wavelength photoacoustic instruments. Simultaneous absorption and scattering measurements in these 32 bands will be enabled by using prisms to (1) spatially separate the super continuum spectrum, (2) modulate each spectral band at an individual acoustic frequency with a custom optical chopper, (3) recombine the spectrum into a laser beam, and (4) send the laser beam through a photoacoustic resonator. Measurements of the scattering coefficient with a scattering sensor in the photoacoustic instrument and the absorption coefficient with the photoacoustic instrument can be achieved for all 32 wavelength bands simultaneously by decoding modulation frequencies with fast Fourier transform (FFT) analysis. This instrument will be used for characterizing carbonaceous aerosol optics and radiative forcing by providing real time, in situ, first principle measurements of aerosol light absorption, scattering, and albedo spectra covering most of the terrestrial solar spectrum.